3D Printing – Terms & Definitions

Tessellated (STL Tessellation)

Made of many small flat shapes (usually triangles) that approximate a curved surface.

DETAIL

When you export a 3D model as an STL, the computer doesn’t store true curves. Instead, every curved surface is broken into many tiny flat triangles.

Those triangles form a tessellation — a tiling of shapes that fit together with no gaps. The smaller and more numerous the triangles, the smoother the surface appears.

This is why zooming in closely on a curved STL shows visible facets.

Full-size graphics for tessellation examples are available in the library.

Parametric (Parametric Modeling)

A modeling approach where shapes are controlled by dimensions and rules instead of fixed geometry.

DETAIL

In a parametric model, sizes such as width, height, wall thickness, hole diameter, or spacing are defined as parameters. Changing a parameter automatically updates the entire model.

This allows one design to generate many variations without redesigning from scratch. Once a model is exported as an STL, it is no longer parametric.

Vase Mode (Spiralize Outer Contour)

A printing mode where the object is printed as one continuous spiral wall with no infill, no top layers, and no seams.

DETAIL

In Vase Mode, the slicer generates a single continuous extrusion path that slowly rises in the Z direction instead of printing discrete layers.

Because only one wall is printed:

- Filament usage is greatly reduced

- Print time is often shorter

- The object is hollow and lightweight

Thermal Sag (Small-Part Heat Deformation)

Gradual leaning or sloping of a printed part caused by heat buildup.

DETAIL

Thermal sag happens when a part does not have enough time to cool between layers. Each new layer reheats the previous ones, leaving the plastic soft.

Gravity and nozzle pressure slowly deform the shape.

Elephant’s Foot

A defect where the bottom of a print flares outward.

DETAIL

Occurs when first layers are squished too much or stay too hot.

Common fixes include adjusting Z offset, flow, and bed temperature.

Part Shift (Adhesion Failure During Print)

A failure where the entire printed part moves on the build plate.

DETAIL

Caused by insufficient bed adhesion resisting lateral forces.

The print continues but becomes offset.