# Prepare Tab

(This page documents everything that happens before slicing: object prep, modifiers, and per-print settings. It is intentionally conceptual-first, not preset-driven.)

## Prepare Tab – Page Map

- Prepare Toolbar

  - Move / Rotate / Scale

  - Measure

  - Cut / Split

  - Paint Support Tool

  - Fuzzy Skin Painting Tool

- Object Types & Modifiers

  - Assembly vs Sub-merge vs Merge into Single Part

  - Fix Model

- Print & Filament Settings (Concepts)

  - Filament settings overview

  - Cooling behavior (layer-time based)

  - Speed, flow, and material intent

- Plate & Object Context (placeholder)

## Print & Filament Settings (Concepts)

(This section explains what filament settings mean, not what numbers to copy. Screenshots are real-world references.)

### Filament Settings – Overview

- Accessed via Filament Settings dialog

- Settings apply per filament profile, not per object

- Changes affect:

  - extrusion behavior

  - cooling behavior

  - material response to geometry

Screenshot references: - 20260106_231359-Filament settings.png - 20260106_231053-Filament settings.png - 20260106_231118-Filament settings.png - 20260106_231149-Filament settings.png - 20260106_231222-Filament settings.png - 20260106_231245-Filament settings.png - 20260106_231314-Filament settings.png

### Cooling (Layer-Time Driven Behavior)

Bambu Studio does not use fixed fan speeds. Cooling is driven by layer time thresholds.

#### Min Fan Speed Threshold

- Defines behavior for very long layers

- Rarely triggered on small or thin parts

- Acts as a failsafe, not a primary control

#### Max Fan Speed Threshold

- Defines the upper fan limit for short layers

- Dominates real-world behavior on:

  - small parts

  - TPU

  - thin walls

Key concept: > Short layers → Max threshold dominates > Long layers → Min threshold may apply

### Cooling Modifiers & Overrides

- Keep fan always on

  - Prevents heat soak and sag on flexible materials

- Slow printing down for better layer cooling

  - Extends layer time instead of increasing fan

- Force cooling for overhangs and bridges

  - Applies extra cooling only where geometry demands it

### Filament Intent vs Geometry

Filament settings must match material behavior, not just printer capability:

- TPU:

  - prefers limited cooling

  - relies on time, not airflow

- PLA:

  - tolerates aggressive cooling

  - benefits from sharp overhang cooling

This explains why identical geometry behaves differently under different filament profiles.

## Plate & Object Context (Placeholder)

(This section will cover per-plate behavior, plate duplication, and object-to-plate relationships.)

# Other Tabs (Placeholders)

## Slice Tab (Placeholder)

- Slicing logic

- Preview modes

- Warnings and analysis

## Device Tab (Placeholder)

- Printer state

- SD card behavior

- Video vs Timelapse

## Project / History (Placeholder)

- Saved plates

- Print history

- Re-slicing behavior

## Prepare Topics

### Object Types & Modifiers

(Authoritative section for how Bambu Studio treats objects, groups, and mesh operations)

### Assembly vs Merged Part vs Sub-merge vs Fix Model

Assembly - Container that groups multiple independent parts - Parts remain separate objects with their own filament, settings, and printable toggles - Best for layouts, multi-color prints, and iterative design

Sub-merge - Merges selected parts within an Assembly into a new merged child - Assembly container remains intact - Other sibling parts stay independent - Best for fusing structural elements while keeping cosmetic or optional parts separate

Merged Part / Merge into Single Part - Boolean-unions all selected parts into one solid object - Loses per-part control and independent toggles - Best for final, locked geometry or single-STL export - Recommended workflow: duplicate first, then merge the copy

Fix Model - Automatic mesh repair tool - Fixes holes, flipped normals, and non-manifold geometry - Does not change layout, dimensions, or part relationships - Best used on imported models or when slicer warnings appear - Safer to run before merging, or on a duplicate for complex geometry

### Prepare Toolbar → Paint Support Tool

What it is - Interactive tool to manually enforce or block support generation on a per-object basis - Only active when an object is selected

Editing Mode Behavior - Entered by clicking the Paint Support icon in the top toolbar - Only the selected object is visible and paintable - Object is rendered in gray for contrast - Manually painted support areas appear in green

Controls & Shortcuts - Left Mouse Button: Enforce supports (painted purple) - Right Mouse Button: Block supports (painted red) - Shift + Left Mouse: Erase painted supports (returns surface to default gray) - Ctrl + Mouse Wheel: Adjust pen size

Color Legend (Paint Support Tool) - Purple: Manually enforced support areas - Red: Manually blocked support areas - Gray: Unpainted / default model surface - Cyan: Build-plate contact area (not a support paint state)

Notes on Colors - Paint Support colors are not user-customizable - Colors are chosen for contrast against the gray editing-mode model - Cyan surfaces indicate plate contact and are independent of support painting

Key Settings - Pen Size: Controls brush diameter - Highlight Overhangs: Visually highlights areas exceeding the support angle threshold - Section View: Allows slicing through the model to paint internal areas - Erase All Painting: Clears all manual support edits

Interaction with Auto Supports - If an auto support type is enabled (e.g., Normal / Auto): - Areas within the support angle threshold are highlighted - Both auto-generated supports and manual painted supports will be generated - Manual painting can override or supplement auto supports

Screenshot Reference - 20260101_202809-PanFlute_v2 - BambuStudio.png

### Prepare Toolbar → Fuzzy Skin Painting Tool

(Manual control of where fuzzy skin is applied on a model)

Purpose - Allows selective application of fuzzy skin to chosen surfaces instead of the entire model - Commonly used for grip areas, decorative texture, or hiding layer lines on visible faces

Tool Modes - Brush: Paint fuzzy skin manually - Fill: Apply fuzzy skin to connected regions based on geometric rules

Smart Fill Angle (Fill Tool) - Defines the maximum surface angle difference that the fill operation will cross - Prevents fuzzy skin from spilling over sharp edges or onto adjacent faces - Lower angle = stricter containment to nearly coplanar surfaces - Higher angle = fill spreads across curved or angled geometry

Practical example: - Low value → only the current face is filled - Medium value → wraps smoothly around cylinders or gentle curves - High value → can cross corners and reach unintended surfaces

Section View - Temporarily slices the model with a movable clipping plane - Allows painting or filling internal or obstructed surfaces - Does not modify the model geometry; it is a visualization and access aid only

Key Notes - Smart Fill Angle affects Fill tool only, not freehand brushing - Section View is especially useful for: - interior walls - recessed grips - overlapping geometry - Fuzzy skin painting applies at slice time and does not alter the STL

Screenshot References - 20260103_163642_fuzzy_BambuStudio_001.png - 20260103_163642_fuzzy_BambuStudio_002.png - 20260103_163642_fuzzy_BambuStudio_003.png - 20260103_163642_fuzzy_BambuStudio_004.png

## Print & Filament Settings (Concepts, Not Presets)

(This section explains what settings mean, not just what values to use. Screenshots are tied to real examples.)

### Filament Cooling → Part Cooling Fan (Example: TPU 95A)

Context - Screenshot: OVERTURE TPU 95A @ Bambu Lab A1 0.4 nozzle - Location: Filament Settings → Cooling - These controls regulate how aggressively the part cooling fan is allowed to run, based on layer time

### Min Fan Speed Threshold

- Fan speed: 100%

- Layer time: 100 s

What this actually means (plain English): - This rule only applies if a layer takes 100 seconds or longer to print - If that happens, the fan may ramp up to 100% - In real prints, especially small or thin TPU parts, layers are far shorter than 100 s

➡️ Practical result: this setting is effectively ignored in most TPU prints and acts as a safety fallback only

### Max Fan Speed Threshold (the one that usually matters)

- Fan speed: 30%

- Layer time: 8 s

What this actually means (plain English): - If a layer prints in 8 seconds or less - The cooling fan is capped at 30% - The fan will not exceed 30% under normal printing conditions

➡️ Practical result: prevents TPU from being over-cooled, stiffening, or splitting at layer lines

### How the Two Thresholds Work Together

- Bambu Studio scales fan speed between these thresholds

- Short layers → limited by Max Fan Speed Threshold

- Extremely long layers → allowed to ramp toward Min Fan Speed Threshold

For TPU: - Short layers are common - Long layers are rare - Therefore Max Fan Speed Threshold dominates behavior

### Key Takeaway

For flexible materials like TPU, the Max Fan Speed Threshold controls almost everything. The Min Fan Speed Threshold is mostly a failsafe for extreme cases.

This is why your TPU seals behaved correctly with the fan capped at ~30%.