# Prepare Topics

## Plates

### Lock Plate

What locking a plate actually does

When a plate is locked in Bambu Studio:

✅ Allowed

- Slice the plate

- Preview toolpaths

- Print the plate

- Change global slicer settings (layer height, infill, etc.)

- View models and settings

🚫 Prevented

- Moving models

- Rotating or scaling models

- Deleting models

- Adding new models to that plate

- Drag-and-drop changes

- Accidental nudges or misclicks

Mental model: Think of it as “freeze this layout”, not “protect the file.”

## Final Navigation Tree (Locked)

OVERVIEW
  Intro & Scope

MAIN MODE TABS
  Prepare
    Overview
    Object Layout & Organization
    Supports & Surface Tools
    Plates & Layout Control
    Print & Filament Settings

  Slice (Preview)
    Overview
    Slice Plate Button
    Preview Colors & Meanings
    Warnings & Errors

  Device
    Overview
    Printer Status & Controls
    Camera, Video & Timelapse
    SD Card & Storage Behavior

  Project
    Overview
    Plates & History
    Saved Projects
    Re-slicing & Versioning

  Calibration
    Overview
    Bed Leveling
    Flow & Pressure Advance
    Material Calibration

MENUS & PREFERENCES
  Preferences
    Overview
    General
    Devices
    Network & Cloud
  File Menu
  View Menu
  Help Menu

KEY FEATURES (Cross-Mode)
  Supports (Concepts)
  Per-Object Settings
  Modifiers
  Ironing
  Timelapse (Concepts)
  Multi-Plate Workflows

TROUBLESHOOTING
  Common Errors
  UI Confusion
  Print Failures
  Camera & Timelapse Issues

REFERENCE
  Terminology
  Screenshot Naming Convention
  Version Notes

Locked rule: Any documentation item must be associated with exactly one Main Mode Tab (Prepare / Slice (Preview) / Device / Project / Calibration). If it spans tabs, assign it to the originating tab and add cross-references as needed.

## Imported Files, Designer Settings, and the Orange Reset Arrow

This section explains how Bambu Studio determines the baseline for settings when you open or import models, and what the orange circular reset arrow is actually comparing against.

### Key rule (locked)

Designer settings only exist when you open a 3MF and allow settings to be imported. Everything else is geometry + your active profiles.

### Case 1: Open a 3MF (Designer Project, Settings Included)

What happens

- Geometry and embedded project settings are loaded

- This may include:

  - print profile overrides

  - filament overrides

  - per-object settings

  - supports and plate layout

Meaning of the orange arrow

- Indicates you changed a value away from the designer’s embedded project setting

- Clicking the arrow resets to the designer’s project value, not your global defaults

#### Orange Arrow Behavior — 3MF with Settings

| Action | What the Arrow Means | What Reset Does |
| --- | --- | --- |
| Change a print setting | You diverged from designer’s project value | Reverts to designer’s value |
| Change a filament setting | You diverged from designer’s filament override | Reverts to designer’s override |
| Switch profiles | May keep overrides if project locks them | Profile defaults may not apply |

Mental model: You are working inside someone else’s recipe.

### Case 2: Open an STL (Geometry Only)

What happens

- Geometry only is imported

- No designer settings exist

- Bambu Studio applies your currently active print and filament profiles

Meaning of the orange arrow

- Indicates you changed a value away from your active profile default

- Clicking the arrow resets to your profile default

#### Orange Arrow Behavior — STL

| Action | What the Arrow Means | What Reset Does |
| --- | --- | --- |
| Change a print setting | You diverged from your print profile | Reverts to profile default |
| Change a filament setting | You diverged from your filament profile | Reverts to filament default |
| Switch profiles | Overrides clear or remap | New profile defaults apply |

Mental model: You brought your own recipe; the model is just the shape.

### Case 3: Import a 3MF and Choose “Geometry Only”

This behaves exactly like an STL, even though the source file is a 3MF.

What happens

- Geometry is imported

- All embedded designer settings are intentionally discarded

Meaning of the orange arrow

- Same as STL: comparison is against your active profiles

#### Orange Arrow Behavior — 3MF (Geometry Only)

| Action | What the Arrow Means | What Reset Does |
| --- | --- | --- |
| Change a setting | You diverged from your profile default | Reverts to your profile default |
| Designer settings | Not present | Cannot be restored |

Mental model: “I want the shape, not the recipe.”

### Unified Mental Model (Hierarchy)

Profile Defaults
  ↳ Designer Project Overrides (3MF only, if imported)
      ↳ Your Local Changes  ← orange arrow lives here

### Where this applies

- Main Mode Tab: Prepare

- Page: Print & Filament Settings (Concepts)

- Cross-reference: Troubleshooting → “Why do I see orange arrows everywhere?”

## Device → Camera, Video & Timelapse

### Live Camera Feed (What the Camera Icon Means)

When the camera icon is lit in the Device page:

- The printer’s camera is powered and actively streaming a live feed

- The feed is available in Bambu Studio and Bambu Handy

Important:

The live camera feed is not recorded and not saved anywhere.

Seeing video only means you are viewing the printer in real time.

### Timelapse (Saved Video)

- Timelapses are separately captured from the live camera feed

- They require:

  - Timelapse enabled

  - A working SD card

  - Supported printer/firmware mode

A timelapse may exist even if you never viewed the live camera, and the live camera may work without producing any saved timelapse.

### Key Distinction (Locked)

- Live camera feed: viewing only, never saved

- Timelapse: intentionally recorded and saved as a video file

Mental model:

Camera icon = “I can see it now” Timelapse = “The printer is saving snapshots to make a video”

### Where This Applies

- Main Mode Tab: Device

- Section: Camera, Video & Timelapse

- Cross-reference: Troubleshooting → Why didn’t my timelapse save?

## Device → Bambu Handy (Mobile App)

### Purpose of Bambu Handy

Bambu Handy is designed as a mobile monitoring and job-control interface, not a full maintenance or configuration tool.

Its focus is:

- starting and stopping prints

- monitoring printer state

- viewing the live camera feed

- checking print history

### What You Can Do in Bambu Handy

- Start, pause, and stop prints

- View current print status and progress

- View live camera feed

- Review completed prints in print history

- View (but not edit) basic printer and filament status

### What You Cannot Do in Bambu Handy

- ❌ Load filament

- ❌ Unload filament

- ❌ Run filament purge or change routines

- ❌ Perform maintenance moves (extrude, retract, jog)

- ❌ Modify print or filament profiles

These actions must be performed:

- on the printer touchscreen, or

- in Bambu Studio (desktop)

### Common Question: Can I unload filament from Bambu Handy?

Answer: No.

Filament load/unload requires:

- nozzle heating

- controlled extrusion and retraction

- physical confirmation at the printer

For safety and reliability reasons, these operations are intentionally not exposed in the mobile app.

### Scope Note (Important)

Bambu Handy should be thought of as:

“A remote window into the printer”

—not a replacement for:

- the printer’s touchscreen, or

- Bambu Studio’s Device controls.

### Where This Applies

- Main Mode Tab: Device

- Section: Bambu Handy (Mobile App)

- Future cross-reference: Handy vs Studio vs Printer capability comparison