# Prepare Tab

(This page documents everything that happens before slicing: object prep, modifiers, and per-print settings. It is intentionally conceptual-first, not preset-driven.)

## Prepare Tab – Page Map

- Prepare Toolbar

  - Move / Rotate / Scale

  - Measure

  - Cut / Split

  - Paint Support Tool

  - Fuzzy Skin Painting Tool

- Object Types & Modifiers

  - Assembly vs Sub-merge vs Merge into Single Part

  - Fix Model

- Print & Filament Settings (Concepts)

  - Filament settings overview

  - Cooling behavior (layer-time based)

  - Speed, flow, and material intent

- Plate & Object Context (placeholder)

## Print & Filament Settings (Concepts)

(This section explains what filament settings mean, not what numbers to copy. Screenshots are real-world references.)

### Filament Settings – Overview

- Accessed via Filament Settings dialog

- Settings apply per filament profile, not per object

- Changes affect:

  - extrusion behavior

  - cooling behavior

  - material response to geometry

Screenshot references: - 20260106_231359-Filament settings.png - 20260106_231053-Filament settings.png - 20260106_231118-Filament settings.png - 20260106_231149-Filament settings.png - 20260106_231222-Filament settings.png - 20260106_231245-Filament settings.png - 20260106_231314-Filament settings.png

### Cooling (Layer-Time Driven Behavior)

Bambu Studio does not use fixed fan speeds. Cooling is driven by layer time thresholds.

#### Min Fan Speed Threshold

- Defines behavior for very long layers

- Rarely triggered on small or thin parts

- Acts as a failsafe, not a primary control

#### Max Fan Speed Threshold

- Defines the upper fan limit for short layers

- Dominates real-world behavior on:

  - small parts

  - TPU

  - thin walls

Key concept: > Short layers → Max threshold dominates > Long layers → Min threshold may apply

### Cooling Modifiers & Overrides

- Keep fan always on

  - Prevents heat soak and sag on flexible materials

- Slow printing down for better layer cooling

  - Extends layer time instead of increasing fan

- Force cooling for overhangs and bridges

  - Applies extra cooling only where geometry demands it

### Filament Intent vs Geometry

Filament settings must match material behavior, not just printer capability:

- TPU:

  - prefers limited cooling

  - relies on time, not airflow

- PLA:

  - tolerates aggressive cooling

  - benefits from sharp overhang cooling

This explains why identical geometry behaves differently under different filament profiles.

## Plate & Object Context (Placeholder)

(This section will cover per-plate behavior, plate duplication, and object-to-plate relationships.)

# Other Tabs (Placeholders)

## Slice Tab (Placeholder)

- Slicing logic

- Preview modes

- Warnings and analysis

## Device Tab (Placeholder)

- Printer state

- SD card behavior

- Video vs Timelapse

## Project / History (Placeholder)

- Saved plates

- Print history

- Re-slicing behavior