# Status

- Source doc received: 20260120_Process_Global_Settings_Screenshots.docx

- Screenshots included: Quality tab + Strength tab (noted: Word inserted them in reverse order).

- Next step: convert these into a clean “Process Settings → Quality / Strength” outline page(s) for the manual.

# Extracted outline (from the provided screenshots)

## Quality tab (screens shown)

### Precision

These settings control how closely the printed geometry matches the digital model, especially for curves, holes, and small dimensional details.

#### Slice gap closing radius

What it affects: Tiny gaps between adjacent extrusion paths.

Increase it when: - You see small pinholes or gaps in top surfaces or thin walls - You want cleaner-looking surfaces on cosmetic parts

Decrease it when: - Fine details are getting blurred or overfilled - Very small features are being lost

Example: A decorative box shows tiny gaps between lines on flat faces → slightly increase this value to visually close them.

#### Resolution

What it affects: How smoothly curves are approximated during slicing.

Increase it when: - Circles or curves look faceted - You’re printing parts with visible curved edges (vases, round housings)

Decrease it when: - Slicing is slow or produces very large G-code files - The model is mostly straight lines or mechanical geometry

Example: A round knob looks slightly polygonal → increase resolution for smoother curves.

#### Arc fitting

What it affects: Converts many short straight moves into true arcs.

Enable it when: - Printing parts with lots of curves - You want smoother motion and smaller G-code files

Disable it when: - Troubleshooting unusual motion behavior - Printing extremely precise mechanical parts where predictability matters

Example: A cylindrical part prints with subtle vibration marks → enabling arc fitting can smooth motion.

#### X–Y hole compensation

What it affects: The printed size of holes.

Increase it when: - Screws, bolts, or pins don’t fit into holes - Holes consistently print undersized

Decrease it when: - Holes are coming out too loose

Example: An M3 screw won’t pass through a printed hole → apply a small positive X–Y hole compensation instead of redesigning the model.

#### X–Y contour compensation

What it affects: The overall external size of the part.

Increase it when: - Parts are consistently slightly too small

Decrease it when: - Parts are consistently slightly too large

Example: A snap-fit box is just a bit too tight overall → a small contour compensation can correct it globally.

#### Auto circle contour–hole compensation

What it affects: Automatically distinguishes between circular holes and outer contours.

Enable it when: - You want better dimensional accuracy without manual tuning - Printing parts with many holes and circular features

Disable it when: - You need full manual control for calibration or testing

Example: A part with many different hole sizes prints inconsistently → enabling this can normalize results.

#### Elephant foot compensation

What it affects: Bottom-layer flare caused by heat and first-layer squish.

Increase it when: - Bottom edges are wider than the rest of the part - Parts don’t sit flat against mating surfaces

Decrease it when: - The bottom edge looks undercut or weak

Example: A box won’t slide into a slot because the bottom edge is too wide → increase elephant foot compensation.

#### Precise Z height

What it affects: Vertical dimensional accuracy.

Enable it when: - Parts must match an exact height - Layer stacking accuracy matters more than speed

Disable it when: - Printing fast prototypes - Minor Z-height variation is acceptable

Example: A printed spacer must be exactly 10 mm tall → enable precise Z height.

### General tuning advice for Precision

- Change one setting at a time when calibrating

- Use small test prints (holes, cubes, rings)

- Prefer compensation settings over modifying the CAD model unless the issue is consistent across printers

### Precision – Quick Reference

| Setting | What it Fixes | Typical Adjustment | When to Touch It |
| --- | --- | --- | --- |
| Slice gap closing radius | Tiny gaps between lines | Small increase | Visible pinholes or gaps on surfaces |
| Resolution | Faceted curves | Increase | Circles or curves look polygonal |
| Arc fitting | Choppy motion on curves | Enable | Parts with many curves |
| X–Y hole compensation | Holes too tight | Small positive value | Screws/pins don’t fit |
| X–Y contour compensation | Part overall size | Small ± adjustment | Whole part too big or small |
| Auto circle contour–hole comp. | Inconsistent holes | Enable | Parts with many holes |
| Elephant foot compensation | Bottom edge flare | Increase gradually | Bottom edge wider than rest |
| Precise Z height | Incorrect height | Enable | Exact Z height required |

### Seam

These settings control where and how each layer starts and ends, which directly affects visible seam lines, surface quality, and strength at layer transitions.

#### Seam position

What it affects: Where the layer start/end seam is placed on the model.

Options explained: - Nearest – Places seams close to the previous layer’s seam; fastest, but may create visible vertical lines. - Aligned – Forces seams into a single vertical line; easy to hide on corners. - Back – Attempts to place seams on the back side of the model relative to the camera. - Random – Distributes seams randomly to avoid a single visible line.

Use it when: - You want predictable seam placement (Aligned / Back) - You want to hide seams visually (Random)

Example: A decorative cylinder shows a single ugly seam → switch from Aligned to Random.

#### Seam placement away from overhangs (experimental)

What it affects: Prevents seams from being placed on overhanging areas where they are more visible or weaker.

Enable it when: - Printing parts with overhangs - Seams look rough or droopy on angled surfaces

Disable it when: - Troubleshooting seam placement - You want full manual control

#### Smart scarf seam application

What it affects: Replaces a hard seam start/stop with a gradual “scarf” transition.

Enable it when: - You want the least visible seams possible - Printing cosmetic or display parts

Disable it when: - Printing fast prototypes - Maximum seam predictability is required

#### Scarf application angle threshold

What it affects: Determines which angles qualify for scarf seam blending.

Increase it when: - You want scarf seams applied more aggressively

Decrease it when: - Seams start affecting sharp features

#### Scarf around entire wall

What it affects: Applies scarf blending around the full wall instead of only at the seam location.

Enable it when: - You want maximum seam hiding

Disable it when: - You want minimal toolpath complexity

#### Scarf steps

What it affects: Controls how gradual the scarf transition is.

Increase it when: - Seams are still visible

Decrease it when: - Surface detail is being softened

#### Scarf joint for inner walls

What it affects: Applies scarf transitions to inner walls as well as outer walls.

Enable it when: - Internal appearance or strength matters

Disable it when: - Inner walls are hidden and speed matters

#### Override filament scarf seam setting

What it affects: Forces scarf seam behavior regardless of filament profile defaults.

Enable it when: - Filament defaults aren’t producing good seam results

#### Role-based wipe speed

What it affects: Adjusts wipe behavior differently for walls, infill, and other roles.

Enable it when: - Seams show blobs or stringing

Disable it when: - Diagnosing extrusion issues

### Seam – Quick Reference

| Setting | What it Fixes | Typical Choice | When to Touch It |
| --- | --- | --- | --- |
| Seam position | Visible seam line | Random / Aligned | Visible vertical seam |
| Away from overhangs | Rough seam on angles | Enable | Seams on overhangs |
| Smart scarf seam | Harsh seam start | Enable | Cosmetic prints |
| Scarf angle threshold | Incomplete blending | Adjust | Scarf not triggering |
| Scarf around entire wall | Visible seam remains | Enable | High-finish parts |
| Scarf steps | Abrupt transition | Increase | Seam still visible |
| Inner wall scarf | Internal seam marks | Enable | Strength/appearance |
| Override filament scarf | Filament mismatch | Enable | Filament-specific issues |
| Role-based wipe speed | Blobs at seam | Enable | Seam blobs/stringing |

### Layer Height

These settings control vertical resolution, surface smoothness, and print time.

#### Layer height

What it affects: Overall print resolution and strength.

Decrease it when: - You want smoother surfaces - Printing detailed or decorative parts

Increase it when: - You want faster prints - Printing large, functional parts

Example: A figurine shows visible layer lines → reduce layer height.

#### Initial layer height

What it affects: First-layer adhesion and bed tolerance.

Increase it when: - Bed leveling is imperfect - You need stronger first-layer adhesion

Decrease it when: - Fine detail is needed on the bottom surface

Example: A part lifts at the corners → increase initial layer height slightly.

### Line Width

These settings control how wide each extrusion line is, affecting strength, surface finish, and dimensional accuracy.

#### Default line width

What it affects: Overall extrusion width for most features.

Increase it when: - You want stronger parts - Slight surface roughness is acceptable

Decrease it when: - You want finer detail

#### Initial layer line width

What it affects: Bed adhesion.

Increase it when: - Prints have adhesion issues

#### Outer wall line width

What it affects: Surface detail and dimensional accuracy.

Decrease it when: - Fine external detail matters

#### Inner wall line width

What it affects: Internal strength.

Increase it when: - Structural strength is important

#### Top surface line width

What it affects: Top surface smoothness.

Decrease it when: - Top surfaces look rough or uneven

#### Sparse infill line width

What it affects: Infill strength and speed.

Increase it when: - Faster prints are preferred over fine infill detail

#### Internal solid infill line width

What it affects: Strength of solid internal layers.

#### Support line width

What it affects: Support strength and removability.

Decrease it when: - Supports are hard to remove