# Bambu Studio – 3MF Printer Profile Behavior

# 1. What is happening

3MF files store printer information along with the model.

When you open a 3MF file, Bambu Studio loads the original printer profile used to create it.

Your current printer is NOT automatically applied.

Result:

- Build plate size may be incorrect

- Purple boundary may not match your printer

- Layout may appear wrong

[Insert Screenshot: mismatched purple build volume boundary]

# 2. How to recognize the problem

Common symptoms:

- Purple box does not match your printer

- Model appears too small or too large

- Printer name at the top does not match your machine

- Plate arrangement looks incorrect

[Insert Screenshot: incorrect printer selected in UI]

# 3. Why Bambu Studio does this

A 3MF file is a complete project file, not just geometry.

It stores:

- Printer profile

- Filament settings

- Plate setup

- Slicing settings

This allows full print setups to be shared between users.

# 4. The quick fix

1. Open the 3MF file

2. Check the selected printer at the top

3. If incorrect, select your printer (e.g., A1)

4. Re-slice the model

[Insert Screenshot: selecting correct printer]

# 5. Clean reset method (recommended)

If things still look wrong:

- Switch to the correct printer

- Delete all plates

- Re-import the model

- Re-slice

This removes hidden mismatches.

[Insert Screenshot: deleting plates / clean workspace]

# 6. Common mistake

Do NOT assume the model is wrong.

In most cases, the issue is simply the printer profile.

# 7. Key takeaway

3MF files load with their original printer profile.

Always verify the printer before printing.