Good. Much better choice building it clean in Fusion 👊

Short answer:

👉 0.5–1.0 mm chamfer is ideal.

Now let’s be specific based on where you’re chamfering.

🔹 Top Edge of the Jar (Outer Rim)

Purpose:

Feels better in hand

Removes sharp edge

Looks finished

Recommendation:

0.75 mm chamfer @ 45°

That’s the sweet spot:

Visually clean

Not fragile

Not bulky

🔹 Bottom Edge (Outside)

Purpose:

Reduces elephant’s foot visibility

Easier print removal

Cleaner look

Recommendation:

0.5 mm chamfer

You don’t need more here.

🔹 Lid Lead-In Chamfer (Very Important)

If lid slides over outside:

Add:

0.5–1.0 mm chamfer on lid inner edge

This helps:

Start the friction fit smoothly

Prevent catching

Prevent shaving plastic

🔹 Insert Hole Edges

For the brush holes:

Add:

0.5 mm chamfer

Just enough to:

Prevent bristle damage

Make insertion smooth

Look intentional

⚠️ Don’t Overdo It

Avoid:

2–3 mm chamfers (too aggressive)

Deep chamfers on thin walls

Chamfering the insert outer edge too much (can loosen fit)

Your Jar (100mm ID, 3mm wall)

Here’s your safe numbers:

Location	Chamfer

Top outer rim	0.75 mm

Bottom outer edge	0.5 mm

Lid inner lead	0.75 mm

Insert holes	0.5 mm

If you want next:

We can dial in the friction fit clearance in Fusion terms so you don’t print three test lids.

it doersn't give me the option for angle

Perfect — that screenshot tells me exactly what’s happening 👌

You’re in Fusion’s Chamfer tool set to “Equal Distance.”

That mode automatically makes a 45° chamfer, which is why you don’t see an angle option.

🔹 Why No Angle Option?

Because:

Equal Distance = Distance1 = Distance2

That mathematically creates a 45° chamfer.

So when you enter:

0.75 mm

You are automatically getting:

0.75 mm @ 45°

You don’t need to specify angle.

🔹 If You Actually Want Angle Control

Change:

Type → Two Distance

Then:

You’ll get Distance1 and Distance2

You can simulate a custom angle

But for printing?

👉 You don’t need it.

Stick with Equal Distance.

For Your Jar

Just:

Select top rim

Equal Distance

Enter 0.75 mm

OK

Done.

If you want, next we can dial in the friction lid tolerance in Fusion terms so you don’t have to print 3 test caps 😄

nevrmind

👍 Got it.

Whenever you're ready to jump back in — whether it's chamfers, friction fit tuning, insert modeling, or something totally different — we’ll pick it up clean.

No FreeCAD trauma required 😄

hw do I check the the size of an object

Fusion or FreeCAD?

Since you just said Fusion, I’ll assume Fusion 360 👊

Short answer:

👉 Use Inspect → Measure

🔹 Fastest Way (Fusion)

Go to the top toolbar

Click Inspect

Click Measure

Click:

An edge (gives length)

Two faces (gives distance)

A circular edge (gives diameter)

Entire body (gives bounding box info)

The Measure panel will show:

Length

Diameter

Distance

Angle

Min / Max / Center distance

🔹 To Get Overall Size (Bounding Box)

Use Measure and:

👉 Click the entire body in the browser tree

or

👉 Click opposite faces

It will show:

X dimension

Y dimension

Z dimension

🔹 If You Want Exact Sketch Dimension

Edit the sketch and:

Click the dimension tool (D)

Click lines or circles

That gives the parametric value.

If you're trying to check:

Overall jar height?

Inner diameter?

Lid clearance?

Insert thickness?

Tell me which one and I’ll give you the cleanest way to check it.

height and diameter of cylinder

Perfect — cylinder is easy 👌

✅ In Fusion 360

1️⃣ Go to Inspect → Measure

🔹 To Check Diameter

Click the circular top edge of the cylinder

The Measure box will show:

Diameter

Radius

If you click the inside edge, you’ll get the inner diameter instead.

🔹 To Check Height

Option A (cleanest):

Click the top flat face

Then click the bottom flat face

It will show the distance (that’s the height)

Option B:

Click one vertical edge

It will show its length

💡 Pro Tip

If it’s hollow:

Click inside top face to inside bottom face to confirm internal height

Or outside faces for total height

If you’re not seeing the values update, tell me:

Are you clicking edges or faces?

Is it a body or still just a sketch?

it's just funny that yhou can't select a solid and see it

😄 Yeah… coming from other CAD or even slicers, that feels weird.

Fusion doesn’t auto-display a “bounding box size” when you click a solid. It assumes:

Sketch = dimension-driven

Bodies = measured on demand

So bodies don’t show size unless you use Inspect → Measure.

Why Fusion Works This Way

Fusion is parametric-first.

If you modeled the cylinder properly:

Height is controlled by the extrude distance

Diameter is controlled by the sketch circle dimension

So instead of “asking the body how big it is,” Fusion expects you to:

Edit the sketch to see diameter

Edit the extrude to see height

That’s the design intent workflow.

Quick Way to See It “At a Glance”

If you want something closer to what you're expecting:

Right-click the body

Choose Properties

It won’t give diameter/height directly, but it gives bounding box extents and volume.

Still not as clean as some CAD packages, I agree.

Honestly though?

If you're building this jar from scratch, the cleanest way to confirm dimensions is:

Edit sketch → see diameter dimension

Edit extrude → see height

That’s the parametric truth.

Want me to show you a fast workflow to make the jar fully parameter-driven so diameter/height are controlled from a single parameter table? That makes this kind of checking almost unnecessary.

trust me, that was the first thing I thought, not problem

😂 I figured that might have been exactly what you did.

You:

Click body

Expect size

Get… nothing useful

Fusion just standing there like “measure it.”

Honestly, once you’re modeling parametric, you almost never measure bodies anyway — you just:

Edit sketch → diameter is right there

Edit extrude → height is right there

So your instinct was correct.

If everything’s behaving now, we’re good.

What’s next — insert sketch time? 👀

I just realized I wante to make this so that the cap and body were flush

Good catch 👊 — that’s a much cleaner look.

You want:

👉 Cap OD = Body OD

So when it’s on, it looks like one continuous cylinder.

That changes the lid design slightly.

How To Do Flush Lid Correctly

Instead of a sleeve that slides over the outside…

You do this:

🔹 Body

Keep body OD = 106 mm

Add a recess at the top inside

Depth: 12–15 mm

Wall thickness stays 3 mm

🔹 Lid

Lid OD = 106 mm (same as body)

Lid has a plug section that fits inside the body

So instead of:

Sleeve over

You do:

Plug inside

Cleaner.

More controlled.

Looks professional.

Recommended Numbers (PLA)

If body inner diameter = 100 mm:

Make lid plug OD:

👉 99.7–99.8 mm

That gives:

0.2–0.3 mm clearance total

Smooth slide fit

If too tight → add +0.05 mm

If too loose → subtract 0.05 mm

Advantages of Flush Design

✔ Looks premium

✔ No double wall at top

✔ Less bulky

✔ Easier to stack visually

✔ Better friction tuning

One Important Detail

Add a tiny chamfer:

0.5 mm on plug edge

0.5 mm on body inner rim

Prevents catching and shaving.

Venting

Still add vent holes to lid top.

Flush doesn’t change airflow need.

Honestly?

Flush plug lid is better than sleeve lid for this project.

If you want, I can give you a quick dimension relationship layout so you don’t accidentally weaken the top wall.

why does it move the grid instead of moveing the object on the grid like a plate

Ahhh yes 😄 — classic Fusion moment.

What’s happening is:

👉 You’re probably panning the view, not moving the body.

Fusion’s grid is just a visual reference tied to the origin.

When you drag with the middle mouse button, you’re moving the camera — not the object.

It looks like the grid is moving, but really the whole scene is.

How To Actually Move the Object

You need to use:

👉 Modify → Move/Copy

Then:

Select the body

Choose Move Type = Translate

Use arrows or type in distance

That moves the object relative to the origin/grid.

Why It Feels Weird

Fusion is origin-centric.

Objects are normally constrained by:

Sketch dimensions

Construction planes

Parametric relationships

So free-dragging parts around isn’t the default behavior.

Quick Tip

If you want the cylinder centered on the grid:

Edit the sketch

Make sure the circle center is constrained to the origin

That’s the clean parametric way.

If you're trying to:

Center the jar?

Raise it off the ground?

Align the lid?

Move it for printing/export?

Tell me which one — different best practice for each.

how do you drop and object to the plane

Good question 👊 — this trips people up.

In Fusion, there isn’t a literal “drop to plane” button like in a slicer. You align it properly.

Here are the clean ways:

✅ If You Want the Bottom Sitting on the XY Plane

Method 1 (Best / Precise)

Go to Modify → Align

Click the bottom face of the cylinder

Then click the XY origin plane

It will snap that face to the plane.

Done.

✅ If You Want to Move It Manually

Modify → Move/Copy

Move Type: Translate

Click bottom face

Move in Z until it hits Z = 0

(or type exact value)

✅ If You’re Building It Parametrically

Even better:

Edit the original sketch

Make sure it was drawn on the XY plane

Make sure the base profile starts at Z = 0

Extrude upward (positive direction)

Then it’s already “on the plane.”

No moving needed.

🔍 How to Check If It’s Floating

Turn on:

👉 Origin in the browser

👉 Show the XY plane

Then inspect from the side.

If there’s a gap, you’ll see it.

If this is for exporting to STL for printing, we can also do the “clean export orientation” version so you don’t accidentally export it floating.

how do you boolean uniton 2 objects

Nice 👊 — now we’re getting into proper CAD moves.

In Fusion 360, Boolean Union is called:

👉 Combine

✅ How to Union (Join) Two Bodies

1️⃣ Go to Modify → Combine

2️⃣ In the dialog:

Target Body → select the main body

Tool Body → select the second body

3️⃣ Operation → choose:

👉 Join

4️⃣ Click OK

Done. They become one body.

🔹 Important Option

There’s a checkbox:

☐ Keep Tools

Leave it unchecked if you want one merged body.

Check it if you want to keep originals.

⚠️ If It Fails

Common reasons:

Bodies are not touching

Bodies overlap weirdly

One is a surface, not a solid

One is inside a different component

If it errors, tell me what the message says.

Are you joining:

Lid plug to lid top?

Insert tabs to body?

Something else?

Different advice depending on which 👀