# Fusion Sketches on Mesh: Why Points Won’t Snap (and How to Fix It)

# The Problem (Real-World Scenario)

You’re working with a mesh model (like a car body), trying to:

- trace an opening (e.g., wheel well)

- create a spline + line

- close the shape so you can extrude

Everything looks correct, but Fusion won’t recognize it as a closed profile.

You may also notice:

- points “snap” but not where you expect

- multiple coincident constraints at endpoints

- the profile only works when the body is hidden

# What You See vs What’s Actually Happening

Visually, your spline appears to follow the wheel well and endpoints appear connected.

In reality, Fusion is snapping to hidden mesh geometry, not what you think you’re clicking.

A common example:

- A center seam from a mirrored model

- Dense mesh vertices along that seam

- Your clicks get pulled there instead of the intended edge

Reference Screenshots:

Closed_Profile_GT40_Example_1.png

Closed_Profile_GT40_Example_3.png

Closed_Profile_GT40_Example_2.png

# Root Causes

1. Mesh Triangulation

Mesh models are made of triangles with lots of tiny edges. Fusion snaps to vertices, not smooth intent.

2. Mirror Seams

Many models are mirrored, creating a perfectly straight centerline seam with dense vertices.

3. Auto-Projected Geometry (Purple Lines)

Fusion may automatically project edges into your sketch, shown in purple. These are unreliable for closed profiles.

4. View-Based Illusion

From one angle it looks correct, from another it is clearly misaligned.

# Key Symptoms (Checklist)

- Profile won’t highlight when hovered

- Multiple coincident constraints on one endpoint

- Points don’t land exactly on visible edges

- Sketch only works when body is hidden

- Top view reveals misaligned points

# The Fix (Step-by-Step)

Step 1 — Hide the Mesh Body

Turn off visibility of the model.

Step 2 — Rebuild the Sketch Clean

Draw spline (4–5 points max), then draw connecting line.

Step 3 — Force Proper Connections

Apply coincident constraint between endpoints.

Step 4 — Test Closure

Hover inside the shape to confirm it fills.

Step 5 — Turn Mesh Back On

# Best Practices (Avoid This Entire Problem)

- Don’t trust snapping on mesh

- Use mesh as a guide only

- Work in multiple views

- Use minimal spline points

- Lock endpoints early

# Why This Happens (Advanced Insight)

Fusion prioritizes snapping to exact vertices first.

Mesh seams create perfect snap targets, so Fusion pulls your clicks toward them even if unintended.

# Quick Rescue Workflow (TL;DR)

1. Hide the body

2. Redraw spline + line

3. Snap endpoints cleanly

4. Test for profile fill

5. Turn body back on

# Key Takeaway

If a sketch won’t close on a mesh, the problem is almost always snapping to the wrong geometry—not the shape itself.

# Final Note

This issue is extremely common with imported or mesh-based models. Once recognized, it becomes quick to fix.